Set-Up Procedure for BlendPro/ScreenPro

Remember this order during setup:

- Projection to BlendPro

- ScreenPro To BlendPro

- Source to ScreenPro

Power-up sequence after units are connected:

- BlendPro – wait for at least 4 dots after the message “Please Wait”

- ScreenPro Plus electronics rack

- ScreenPro plus control head

After turning BlendPro to (on) make sure you:

- Reset unit to Factory default

- Set serial control to RS-485

- Set ID to #5 (1 thru 4) are already used by the ScreenPro

- Make sure to save new settings on BlendPro

After turning ScreenPro on:

- Make sure you do a “Reset All” on Control Head

- Control Head should read BP-0 in the top right corner of display

- Turn Control Head off/on and unit should read BP-1

- Make sure the Display Lock is turned on

# Go to BlendWizard program on computer and start at the planning set-up Tab

To determine projection positions:

- Set screen dimensions

- Hit calculate and GUI will show what the overlap value is

- Set overlap – we generally like 256 as a standard, but not always written in stone.

- After you do all the settings the display in the bottom right will give you the Centering locations for the projectors

*Projectionist must use internal grid to align projectors geometrically.

Overlap average on 1280 x 1024 with 256 overlap should be 3 boxes on Barco projectors.

To set Projection to BlendPro:

- After Projectionist has done geometry with the internal grid

- Go to BlendPro and use the Projection setup tab.

- Turn on the 1on/1off pattern

- Have the projectionist do his file setup for position and phasing making sure position is first pixel and last pixel Horizontally

- Then turn off 1on/1off pattern and turn on Alignment pattern

- Have projectionist check alignment of overlap using the lens shift of the projector

- Now turn on the Output Raster Box

- Once again check for Alignment as well as geometry of overall image

- Now turn off ALL patterns and turn on HRAMP pattern

- Now projectionist can do Color Balance

To Setup ScreenPro to BlendPro:

- Turn test pattern on from ScreenPro to BlendPro.  *Note: Dual input CRT’s are used as operation Preview & BlendPro output reference so that you can better judge the phasing issue. LCD’s are not very friendly here.

- Test Pattern from ScreenPro should be set to Pixel on/off (or Burst)

- On Laptop, Go to GUI and click on source-set-up.

- Adjust phase until monitors look their Brightest.  Don’t go to the Dark side!  It is best to go the full range of the phase to make sure.

- After all three sources are phased, save all 3 configurations.  Click (Save all)

- Next test Pattern from ScreenPro to BlendPro should be H-Ramp.

- Manually go to BlendPro unit and execute an AUTO color balance on BlendPro.

- Process takes a few minutes

- Go to Serial Port on GUI and click on “Read from BlendPro.”

- Go to display Set-Up on GUI and click on system save BlendPro.

- Go to source set-up on GUI

- With ScreenPro sending Pixel on/off (or Burst) with AR Box on.

- This process makes sure your outputting the correct Pixel position to BlendPro.

- Go to GUI and make sure Data Doubling is on.

- Make sure seam removal is in off position on all 3 Screens on BlendPro.

- On GUI go to source set-up and adjust horizontal on all three inputs.

- Separate 1 & 3 from 2.  Make sure #2 has lines all around.  You should be able to offset Horizontal plus or Minus 1 to see line of AR box disappear in both directions.

- After 2 is set move 1 & 3 to butt up to 2. Boxes on either side should not over lap

- On ScreenPro set Test pattern to H-Ramp look at BlendPro output on CRT and on GUI make sure H-Ramp of ScreenPro is even across blended area using the “Overall Contrast” as well as the “Overall Brightness” to achieve even looking Seam.  *Note: this process helps immensely in subtracting the Seams visible on CRT.

- When you feel you have matched Brightness and Contrast across all SEAMS do an all channel SAVE configuration

To start setting up Source to ScreenPro (This setup is Primarily for the Palette or Canvas Computer(s)):

- Turn Test Pattern off on ScreenPro

- Set PVW and PGM Inputs to be the same on all screens. *I.e.: Screen 1 should have input 1; Screen 2 should have input 2; and screen 3 should have input 3 on PVW and PGM

- Computer which is set at 3072 x 768 should have several test pattern alignments on Desktop.  If not Check BTTB Website for them.

- Setup the Display Properties background of computer to be 1on/1off pattern for screen resolution (1024x768 or 1280x1024)

- Go to preview of each individual channels and do an autoconfig *Note: The AR Box on the ScreenPro should be off.

- Check H-Total on input (1344 for 1024x768 and 1688 for 1280x1024)

- Turn AR Box on to align H-Pos & V Pos of source. Once again check that the First pixel and the Last pixel are visible and Match the AR Box. *Note: Save Constantly.

- Phase Adjust all Channels on Preview then Transition Preview to Program and do it once more.  Note: You must Phase both Scalers per channel

- On GUI go to source set up

- Source (computer) should send an H-Ramp pattern from desktop.

- Go to seam removal on GUI and turn seam Bridge on, on all 3 channels.  Make sure Bridge Threshold is set to 255.

- Look at source on output of BlendPro with CRT. And see of any final adjustments are needed. Adjust brightness & contrast in source set up to remove lines on screen.

- Process should be done.

- Small fixes include -  On # 2 source in of BlendPro you might adjust phase and offset H position of #2 on BlendPro GUI