Eagle 200TM External Control Protocol

FSR Inc.

244 Bergen Boulevard

West Paterson, NJ 07424

Phone (973) 785-4347 • Fax (973) 785-4207

# Revision history

| Revision | Date | Description | Initials |
| --- | --- | --- | --- |
| 1.00 | 3/1/08 | First Release |  |

TABLE OF CONTENTS

Syntax	4

Serial Parameters	4

RS232 Mode	4

Ethernet Parameters	4

COMMANDS	5

KP [key label] <button action>	5

# Syntax

## Serial Parameters

The following are the default parameter settings for serial communication.

Baud Rate is 115.2K.

Parity is NONE.

Stop Bit is 1.

Data Bit is 8.

## RS232 Mode

The Eagle 200 will respond with a '#' prompt when the command processor is ready for a command.

The command syntax is shown below

cmd arg1 arg2 … argn<CR>

| cmd | cmd is any valid command, typically 2 to 6 alphabetic (non numeric) characters. |
| --- | --- |
| arg | arg1, arg2, … argn are required or optional parameters depending on the command used. |
| <CR> | carriage return (ASCII 13) terminates the command |

A space (ASCII 32) must be inserted between the command and any arguments that follow. A space must also be inserted between all argument parameters except for the last argument in the chain.

All commands in RS232 mode must be terminated with a carriage return (ASCII 13). The carriage return will tell the command processor to begin execution of the command. See below for a description of error codes.

Query commands will return the following:

=result

#

The '=' indicates a result from a command is following.  The 'result' will follow directly after the '='.  The value of the result will vary depending on the query command used.  A new line will be generated and the prompt will indicate the system is ready for a new command.

Commands to Eagle 200 may be concatenated provided that each command is separated with a carriage return (0x0D or ASCII 13) and no more than 3072 characters are sent at a time.

## Ethernet Parameters

The following information describes how to connect to Eagle 200 via Ethernet.

Communication Protocol Supported: TELNET

Default IP Address: 192.168.0.10

Communication Port: 3000

Example: telnet 192.168.0.10 3000 (This example can be done from a Command Prompt in Windows).

Once connected to Eagle 200, operation is the same as in RS232 Mode.

# COMMANDS

Command:

## KP [key label] <button action>

Description: 	This command emulates key presses for the front panel of the Eagle 200.

Parameters:	key label [see Table Below]

button action [H|R|D] Key Hold | Key Release | Key Double Punch

Example:	KP 1	(Performs a key press/release function on the Source 1 key.)

KP MOVE D  (Performs a Double Punch on the Move key,)

[key label] – This parameter specifies the button to push and/or release.

Eagle 200 II Key Label:

| Actual Button Text | [key label] |
| --- | --- |
| 1 | 1 |
| 2 | 2 |
| 3 | 3 |
| 4 | 4 |
| 5 | 5 |
| 6 | 6 |
| 7 | 7 |
| 8 | 8 |
| SDI 1 | SDI1 |
| SDI 2 | SDI2 |
| BG A | BGA |
| BG B | BGB |
| Layer A | LAYERA |
| Layer B | LAYERB |
| DSK | DSK |
| Logo | LOGO |
| Wipe | WIPE |
| Cut | CUT |
| Mix | MIX |
| Split Layer | SPLITLAYER |
| Key | KEY |
| Full Screen | FULLSCREEN |
| PIP | PIP |
| Clear Layer | CLEARLAYER |
| Swap Z-Order | SWAPZORDER |
| Freeze | FREEZE |
| Reset | RESET |
| Move Setup | MOVESETUP |
| Move | MOVE |

<button action> - This is an optional parameter that will perform as follows:

Not Specified – Key Label will act as if the button has been pushed and then immediately released.

H – Key label will act as if the button is being held in a pushed state.

R – Key label will act as if the button is being released, only if currently in a held state.

D – Key label will act as if the button is being double-punched.